## ASSIGNMENT NO # 02

NAME:  NIMRAH WASEEM.

REG NO: SP23-BCS-108.

SECTION: C.

DATE: 25-09-2025

## Conceptual Question

• Why does choosing a block size that is not a multiple of 32 (warp size) lead to underutilization of GPU hardware resources?

GPUs execute threads in groups of 32 called warps. If a block size is not a multiple of 32, some threads in the last warp remain unused. This results in idle threads that waste GPU resources and reduce overall throughput.

• Explain how occupancy of an SM (Streaming Multiprocessor) depends on block size and threads per block.

Occupancy refers to the ratio of active warps per SM to the maximum possible warps. It depends on:
  - The number of threads per block (larger blocks may increase or decrease occupancy depending on resource usage).
  - Resource usage per thread (registers, shared memory).
  - GPU hardware limits (max threads per SM, max blocks per SM).

## Analysis Question

• Suppose you run an image filter with the following configurations:
  o Case A: 64 threads per block
  o Case B: 256 threads per block
  o Case C: 1024 threads per block
• If Case B is fastest, explain why neither the smallest nor the largest block size gave optimal performance.

With 64 threads per block, there is underutilization because too few threads are scheduled per block, leading to low occupancy. With 1024 threads per block, resource limits like registers and shared memory per block may prevent multiple blocks from being scheduled on an SM, reducing parallelism. 256 threads per block provides a balance between occupancy and resource usage, leading to the best performance.

## Discussion Question

• Why does increasing the number of threads per block not always improve performance?

While more threads per block can increase parallelism, performance does not always improve due to:
  - Register pressure: Each thread consumes registers, and too many threads may exceed hardware limits.
  - Shared memory limits: Larger blocks require more shared memory, limiting the number of concurrent blocks.
  - Scheduling constraints: GPUs may schedule fewer blocks if each block is too large, reducing overall occupancy.